# Gersemi Changes

# TL012 changes

- Check with camera, if possible, where the line might be touching (upstream and downstream)

- Remove the outer pipe from the bellow to the bayonet connection

- Add MLI

- Add new copper shield

- Add new outer pipe

- Add temperature sensor if possible

Add wider outer tube

Add TT

# VCS changes

- Move sensors from tower on top of cryostat to a side flange on the vacuum vessel (to be able to lift the pressure vessel without having to disconnect these sensors). Mostly these are for the LN2 shield

- Lift the pressure vessel and check/fix the sensors that are broken, i.e. TT743B, TT743C, TT743D…

# VB changes

- Add dynamic sensor from IPNO

- Exchange inlet line to HX600

- Check wiring of TT610 and TT611. It seems that almost always TT610 > TT611.

- Check EH601 and TT601 (sensor increases temperature very slowly when heater is on)

# Discussions on 18th May 2021

- To open up TL012 next Monday 24th and check connections on the upper lines to the copper shield and of CV601 to any other line, and check if there is a copper shield on the lower part.

- Wait for input from above before deciding what to do with TL012: repair or replace

- Not add a temperature sensor on the upper part, cables too fragile to make it through the o-ring unscathed.